Automatic Washing Machine Controller

Abstract

An automatic washing machine controller is a digital control system designed to manage the different stages of a washing process automatically. The controller performs operations such as filling water, washing, rinsing, draining, and spinning without manual intervention. In this project, the controller is implemented using a Finite State Machine (FSM) designed in Verilog Hardware Description Language (HDL). The system receives inputs from sensors such as the start button, door sensor, and water level sensor and produces outputs to control components like the motor, water valve, drain valve, and buzzer. The design is simulated using Vivado simulation tools to verify correct operation. This project demonstrates the practical application of digital logic design and FSM concepts in real-world household appliances.

Introduction

Modern washing machines use embedded digital controllers to automate the washing process. The washing operation consists of several stages including water filling, washing, draining, rinsing, and spinning. These stages must occur in a specific order for proper operation.

A Finite State Machine (FSM) is an ideal approach to control sequential operations in digital systems. FSM divides the system into different states and transitions between them based on inputs and conditions.

In this project, an FSM-based washing machine controller is designed using Verilog HDL. The system starts when the user presses the start button and the door is closed. The controller then performs each washing stage sequentially. The design is tested and verified through simulation in Vivado.

Block Diagram

The washing machine controller consists of three major blocks.

1. Input Section

- Start Button

- Water Level Sensor

- Door Sensor

- Clock

- Reset

2. Control Unit

- Finite State Machine Controller

- Controls state transitions

- Generates control signals

3. Output Section

- Motor Control

- Water Valve

- Drain Valve

- Buzzer

Data Flow

Inputs (Sensors & Switches)
        ↓
FSM Controller
        ↓
Control Signals
        ↓
Motor / Water Valve / Drain / Buzzer

4. FSM Diagram

The washing machine operates in six states.

1. IDLE

The machine waits for the user to press the start button. It also checks whether the door is properly closed.

2. WATER FILL

The water valve is turned ON and the machine fills water until the water level sensor indicates that the tank is full.

3. WASH

The motor rotates the drum to wash the clothes.

4. DRAIN

The drain valve opens and removes dirty water from the machine.

5. RINSE

Fresh water is added and the drum rotates again to rinse the clothes.

6. SPIN

The motor spins at high speed to remove water from clothes.

After completion, the system returns to the IDLE state.

State Flow

IDLE → FILL → WASH → DRAIN → RINSE → SPIN → IDLE

5. Verilog Code

module washing_machine_controller(
input clk,
input reset,
input start,
input water_level,
input door_closed,
output reg motor,
output reg water_valve,
output reg drain,
output reg buzzer
);

parameter IDLE  = 3'b000;
parameter FILL  = 3'b001;
parameter WASH  = 3'b010;
parameter DRAIN = 3'b011;
parameter RINSE = 3'b100;
parameter SPIN  = 3'b101;

reg [2:0] current_state, next_state;

always @(posedge clk or posedge reset)
begin
if(reset)
current_state <= IDLE;
else
current_state <= next_state;
end

always @(*)
begin
case(current_state)

IDLE:
if(start && door_closed)
next_state = FILL;
else
next_state = IDLE;

FILL:
if(water_level)
next_state = WASH;
else
next_state = FILL;

WASH: next_state = DRAIN;
DRAIN: next_state = RINSE;
RINSE: next_state = SPIN;
SPIN: next_state = IDLE;

default: next_state = IDLE;

endcase
end

always @(*)
begin
motor = 0;
water_valve = 0;
drain = 0;
buzzer = 0;

case(current_state)

IDLE: begin end

FILL: water_valve = 1;

WASH: motor = 1;

DRAIN: drain = 1;

RINSE:
begin
motor = 1;
water_valve = 1;
end

SPIN:
begin
motor = 1;
buzzer = 1;
end

endcase
end

endmodule

Testbench

module washing_machine_tb;

reg clk;
reg reset;
reg start;
reg water_level;
reg door_closed;

wire motor;
wire water_valve;
wire drain;
wire buzzer;

washing_machine_controller uut(
.clk(clk),
.reset(reset),
.start(start),
.water_level(water_level),
.door_closed(door_closed),
.motor(motor),
.water_valve(water_valve),
.drain(drain),
.buzzer(buzzer)
);

always #5 clk = ~clk;

initial
begin

clk = 0;
reset = 1;
start = 0;
water_level = 0;
door_closed = 1;

#10 reset = 0;
#10 start = 1;

#20 water_level = 1;

#40 water_level = 0;

#200 $finish;

end

endmodule

Simulation Results

- Machine starts in IDLE state

- Water valve activates during FILL state

- Motor operates during WASH, RINSE, and SPIN states

- Drain valve activates during DRAIN state

- Buzzer activates after SPIN completion

State Transition Observed

IDLE → FILL → WASH → DRAIN → RINSE → SPIN → IDLE

Advantages

- Fully automated washing process

- Reduces human effort

- Efficient water usage

- Reliable sequential control using FSM

- Can be implemented on FPGA hardware

Applications

- Household washing machines

- Industrial laundry automation systems

- Smart home appliances

- Embedded system controllers

- FPGA based automation systems

Conclusion

The Automatic Washing Machine Controller was successfully designed using a Finite State Machine (FSM) in Verilog HDL. The system effectively controls different washing stages such as filling, washing, draining, rinsing, and spinning. Simulation results confirm the correct operation of the controller. This project demonstrates the practical application of digital design concepts in real-world appliances and is suitable for implementation on FPGA platforms using tools like Vivado.

Simulation Values

1. Input Values Used in Testbench

| Time (ns) | clk | reset | start | water_level | door_closed | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 | 1 | System Reset |
| 10 | 1 | 0 | 0 | 0 | 1 | Reset Released |
| 20 | 0 | 0 | 1 | 0 | 1 | Start Button Pressed |
| 40 | 1 | 0 | 1 | 1 | 1 | Water Tank Full |
| 80 | 0 | 0 | 1 | 0 | 1 | Washing Process |
| 120 | 1 | 0 | 1 | 0 | 1 | Draining |
| 160 | 0 | 0 | 1 | 0 | 1 | Rinsing |
| 200 | 1 | 0 | 1 | 0 | 1 | Spinning |

2. FSM State Simulation

| Time (ns) | Current State | Motor | Water Valve | Drain | Buzzer |
| --- | --- | --- | --- | --- | --- |
| 0 | IDLE | 0 | 0 | 0 | 0 |
| 20 | FILL | 0 | 1 | 0 | 0 |
| 40 | WASH | 1 | 0 | 0 | 0 |
| 80 | DRAIN | 0 | 0 | 1 | 0 |
| 120 | RINSE | 1 | 1 | 0 | 0 |
| 160 | SPIN | 1 | 0 | 0 | 1 |
| 200 | IDLE | 0 | 0 | 0 | 0 |

3. Expected Output Behavior

Water Fill State

water_valve = 1
motor = 0
drain = 0
buzzer = 0

Washing State

motor = 1
water_valve = 0
drain = 0
buzzer = 0

Drain State

drain = 1
motor = 0
water_valve = 0
buzzer = 0

Rinse State

motor = 1
water_valve = 1
drain = 0
buzzer = 0

Spin State

motor = 1
buzzer = 1